# Outer and Middle Ear Simulator Code Appendix

Ready to upload to Google Docs and attach to the assignment

This appendix documents the browser-based simulation used for the outer-ear and middle-ear assignment. It explains the circuit verification method, records the required experiments and model assumptions, provides spaces for website screenshots, and includes the complete source code used by the simulator.

The class-supplied equivalent circuit is the source of truth. The current code makes its provisional topology explicit so that the series and parallel connections can be checked rather than accepted automatically from an AI suggestion.

# Submission Checklist

| Requirement | What to include |
| --- | --- |
| Code | Attach this appendix or include the complete source-code sections at the end of the Google Doc. |
| Plots | Paste screenshots of the outer-ear, middle-ear, combined, phase, input-impedance, and sweep plots from the website. |
| Experiments | Report the prediction and observed result for +20% inertance, +20% compliance, and +20% resistance. |
| Topology | Show the class circuit, label each series/parallel connection, and compare it with the equations in this appendix. |
| Parameter source | Identify at least three parameters as derived, measured/fitted, or assumed. |
| Limitations | State at least three limitations of the combined model. |
| AI use | Explain how GPT-5.6 helped and document one suggestion that was corrected or rejected after checking the physics. |

# Circuit Verification

Use the following procedure in the Google Doc. It demonstrates that the circuit was checked independently of the AI-generated code.

1. Redraw the exact class circuit by hand or insert a photograph of it. Mark the input node, the output node at the cochlear load, every junction, and the reference node.

2. Label every element as a resistor, inertance, or compliance. Write Z_R = R, Z_L = j omega L, Z_C = 1/(j omega C), and omega = 2 pi f beside the circuit.

3. Identify series elements by checking whether the same current must pass through them. Identify parallel elements by checking whether they share the same two nodes. Do not decide series or parallel from the subsystem names alone.

4. Compare the drawing with the middle-ear functions in app.js: Z_cavity, Z_eardrum, Z_ossicles, Z_joint, and Z_cochlea. Check that each function corresponds to the element or subsystem shown in the class diagram.

5. Check the combination step in computeMiddleEarResponse. The provisional implementation forms Z_series = Z_eardrum + Z_ossicles + Z_joint + Z_cochlea and then forms Z_in = Z_series || Z_cavity. Replace this step if the class circuit uses a different topology.

6. Perform one hand calculation at a test frequency such as 1 kHz. Calculate the complex impedance of each element, combine the values using the class topology, and compare the result with the app's input impedance at the same frequency.

7. Run limiting-case checks. A branch that is disconnected should have no effect, a very large series resistance should reduce transmission, and moving an L or C value should move the resonance in the direction predicted by f0 = 1/(2 pi sqrt(LC)).

8. Record the final topology in one sentence and state that it was verified against the class diagram before the final results were accepted.

## Topology Statement

The current implementation assumes a cavity shunt across a series eardrum -> ossicles -> incudo-stapedial joint -> cochlear-load path. The input impedance is Z_in = Z_series || Z_cavity. This is a provisional statement until it has been compared with the supplied class diagram.

# Plot Screenshots

Paste screenshots from the simulator below. Keep the plot title, axes, and visible parameter settings in each image.

## Figure 1 Outer-ear response alone

Paste website screenshot here.

## Figure 2 Middle-ear transmission magnitude

Paste website screenshot here.

## Figure 3 Middle-ear transmission phase

Paste website screenshot here.

## Figure 4 Middle-ear input impedance

Paste website screenshot here.

## Figure 5 Combined outer plus middle response

Paste website screenshot here.

## Figure 6 Real-time parameter sweep

Paste website screenshot here.

# Three Parameter Experiments

The simulator changes one middle-ear parameter by 20 percent from the current baseline. Paste the exact peak frequencies and level changes shown in the experiment cards, then state whether the prediction matched the result.

| Change | Prediction | Observed result | Match |
| --- | --- | --- | --- |
| Eardrum and ossicle inertance +20% | The resonance should move lower because f0 decreases as inertance increases. | Paste the app result here. | Yes or no |
| Eardrum compliance +20% | The resonance should move lower because f0 decreases as compliance increases. | Paste the app result here. | Yes or no |
| Eardrum and joint resistance +20% | Loss should increase, reducing the sharpness and usually lowering the peak magnitude. | Paste the app result here. | Yes or no |

# Parameter Provenance

Do not call a value measured or fitted unless it came from a cited source or a calibration dataset. The normalized middle-ear values in this model are assumptions used to demonstrate sensitivity.

| Parameter | Category | Reason | Source or note |
| --- | --- | --- | --- |
| Ear-canal length | Physically interpretable and assumed baseline | Used in f_n = n c/(4 L_eff). | Baseline value is a model choice unless replaced by a measurement. |
| Concha volume and aperture | Physically interpretable and assumed baseline | Used in the Helmholtz relationship. | Use measured anatomy only if a source is cited. |
| Middle-ear inertance | Assumed or fitted model parameter | Controls the L term in Z_L = j omega L. | The app labels it normalized model units. |
| Middle-ear compliance | Assumed or fitted model parameter | Controls the C term in Z_C = 1/(j omega C). | Replace with a cited value if available. |
| Middle-ear resistance and cochlear load | Assumed or fitted model parameters | Represent losses and loading in the lumped network. | Not clinical measurements in this simulator. |

# Model Limitations

1. The middle-ear circuit is a simplified lumped network and may not include every series, parallel, transformer, or area-ratio detail in a full middle-ear model.

2. The cascade H_total = H_outer x H_middle assumes that the middle-ear input impedance does not load or alter the outer-ear transfer function.

3. The model is not subject-specific and does not represent three-dimensional sound radiation, individual HRTFs, nonlinear tissue behavior, or clinical diagnosis.

# AI Use and Correction

GPT-5.6 in Codex was used as a coding copilot to translate the impedance equations into JavaScript, build the plots and controls, and generate the experiment summaries. The physics and circuit topology were not treated as automatically correct.

Example correction to document after comparing with the class diagram: the AI-generated model used a provisional cavity-shunt and series-path interpretation. I checked the class drawing node by node. If the class diagram shows a different connection, I rejected the provisional combination, changed computeMiddleEarResponse, and reran the impedance and transmission plots. This correction is important because the same individual impedances can produce a different total response when their series and parallel connections change.

Replace the example wording with the exact correction you made after checking your own class circuit. Do not claim that a topology was verified if you have not compared it with the supplied diagram.

# Code Appendix

The following listings are the complete browser implementation. The files should be submitted with the PDF or included in the appendix if the instructor wants the source inside the document.

## index.html

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="UTF-8" />

<meta name="viewport" content="width=device-width, initial-scale=1.0" />

<title>Outer + Middle Ear Acoustics Lab</title>

<link rel="stylesheet" href="styles.css" />

</head>

<body>

<div class="page-shell">

<header class="hero panel">

<div class="hero__copy">

<p class="eyebrow">Outer + Middle Ear Acoustics Lab</p>

<h1>Interactive anatomy-to-cochlear-load transfer model</h1>

<p class="hero__lede">

Follow the signal from pinna and concha through the ear canal,

eardrum, ossicles, and cochlear load with adjustable acoustic and

mechanical parameters.

</p>

</div>

<div class="hero__stats">

<div class="stat-card">

<span class="stat-card__label">Model span</span>

<strong>20 Hz to 20 kHz</strong>

</div>

<div class="stat-card">

<span class="stat-card__label">Signal path</span>

<strong>Pinna to cochlea</strong>

</div>

<div class="stat-card">

<span class="stat-card__label">Preview mode</span>

<strong>Live plots + audio</strong>

</div>

</div>

</header>

<main class="layout">

<aside class="panel controls">

<div class="section-heading">

<h2>Biological Controls</h2>

<p>Adjust the anatomy and watch the transfer function update.</p>

</div>

<section class="control-group">

<div class="control-group__heading">

<h3>Ear Canal</h3>

<p>Quarter-wave modes and simplified IEC 318 branches.</p>

</div>

<label class="control">

<div class="control__top">

<span>Canal length</span>

<output id="canalLengthMmValue">27.0 mm</output>

</div>

<input

type="range"

min="20"

max="40"

step="0.5"

value="27"

data-param="canalLengthMm"

/>

</label>

<label class="control">

<div class="control__top">

<span>Canal diameter</span>

<output id="canalDiameterMmValue">7.0 mm</output>

</div>

<input

type="range"

min="4"

max="10"

step="0.1"

value="7"

data-param="canalDiameterMm"

/>

</label>

<label class="toggle">

<input type="checkbox" id="iec318Enabled" checked />

<span>Include simplified IEC 318 equivalent-circuit branches</span>

</label>

</section>

<section class="control-group control-group--middle">

<div class="control-group__heading">

<h3>Middle Ear</h3>

<p>Normalized lumped impedances for the eardrum, ossicles, joint, cavity, and cochlear load.</p>

</div>

<label class="control">

<div class="control__top">

<span>Eardrum + ossicle inertance</span>

<output id="middleInertanceValue">0.0012 m.u.</output>

</div>

<input

type="range"

min="0.0004"

max="0.0030"

step="0.0001"

value="0.0012"

data-param="middleInertance"

/>

</label>

<label class="control">

<div class="control__top">

<span>Eardrum compliance</span>

<output id="middleComplianceValue">0.000020 m.u.</output>

</div>

<input

type="range"

min="0.000008"

max="0.000060"

step="0.000001"

value="0.000020"

data-param="middleCompliance"

/>

</label>

<label class="control">

<div class="control__top">

<span>Eardrum/joint resistance</span>

<output id="middleResistanceValue">0.65 m.u.</output>

</div>

<input

type="range"

min="0.10"

max="2.00"

step="0.05"

value="0.65"

data-param="middleResistance"

/>

</label>

<label class="control">

<div class="control__top">

<span>Middle-ear cavity volume</span>

<output id="middleCavityVolumeCcValue">1.5 cc</output>

</div>

<input

type="range"

min="0.5"

max="3.0"

step="0.1"

value="1.5"

data-param="middleCavityVolumeCc"

/>

</label>

<label class="control">

<div class="control__top">

<span>Incudo-stapedial compliance</span>

<output id="jointComplianceValue">0.000035 m.u.</output>

</div>

<input

type="range"

min="0.000012"

max="0.000080"

step="0.000001"

value="0.000035"

data-param="jointCompliance"

/>

</label>

<label class="control">

<div class="control__top">

<span>Cochlear load resistance</span>

<output id="cochlearLoadValue">1.80 m.u.</output>

</div>

<input

type="range"

min="0.50"

max="4.00"

step="0.10"

value="1.80"

data-param="cochlearLoad"

/>

</label>

<p class="microcopy topology-note">

Topology assumption to verify against class: cavity shunt across

a series eardrum -> ossicles -> joint -> cochlear-load path.

</p>

</section>

<section class="control-group">

<div class="control-group__heading">

<h3>Concha Cavity</h3>

<p>Helmholtz resonance from volume and aperture geometry.</p>

</div>

<label class="control">

<div class="control__top">

<span>Concha depth</span>

<output id="conchaDepthMmValue">12.0 mm</output>

</div>

<input

type="range"

min="5"

max="20"

step="0.5"

value="12"

data-param="conchaDepthMm"

/>

</label>

<label class="control">

<div class="control__top">

<span>Concha volume</span>

<output id="conchaVolumeCcValue">3.0 cc</output>

</div>

<input

type="range"

min="1"

max="8"

step="0.1"

value="3"

data-param="conchaVolumeCc"

/>

</label>

<label class="control">

<div class="control__top">

<span>Aperture radius</span>

<output id="conchaApertureMmValue">4.5 mm</output>

</div>

<input

type="range"

min="2"

max="8"

step="0.1"

value="4.5"

data-param="conchaApertureMm"

/>

</label>

</section>

<section class="control-group">

<div class="control-group__heading">

<h3>Concha Reflector</h3>

<p>Parabolic-reflector approximation with a 2 kHz-ish break.</p>

</div>

<label class="control">

<div class="control__top">

<span>Reflector length L</span>

<output id="reflectorLengthMmValue">5.0 mm</output>

</div>

<input

type="range"

min="2"

max="12"

step="0.2"

value="5"

data-param="reflectorLengthMm"

/>

</label>

<label class="control">

<div class="control__top">

<span>Reflector radius a</span>

<output id="reflectorRadiusMmValue">10.0 mm</output>

</div>

<input

type="range"

min="5"

max="18"

step="0.2"

value="10"

data-param="reflectorRadiusMm"

/>

</label>

</section>

<section class="control-group">

<div class="control-group__heading">

<h3>Pinna Reflection</h3>

<p>Direction-dependent path difference and destructive interference.</p>

</div>

<label class="control">

<div class="control__top">

<span>Base reflection path</span>

<output id="reflectionPathMmValue">14.0 mm</output>

</div>

<input

type="range"

min="8"

max="24"

step="0.2"

value="14"

data-param="reflectionPathMm"

/>

</label>

<label class="control">

<div class="control__top">

<span>Reflection strength</span>

<output id="reflectionStrengthValue">0.60</output>

</div>

<input

type="range"

min="0.05"

max="0.95"

step="0.01"

value="0.6"

data-param="reflectionStrength"

/>

</label>

<label class="control">

<div class="control__top">

<span>Source azimuth</span>

<output id="azimuthDegValue">0 deg</output>

</div>

<input

type="range"

min="-90"

max="90"

step="1"

value="0"

data-param="azimuthDeg"

/>

</label>

<label class="control">

<div class="control__top">

<span>Source elevation</span>

<output id="elevationDegValue">0 deg</output>

</div>

<input

type="range"

min="-45"

max="90"

step="1"

value="0"

data-param="elevationDeg"

/>

</label>

</section>

<section class="control-group">

<div class="control-group__heading">

<h3>Actions</h3>

<p>Reset values or export a report-ready summary snippet.</p>

</div>

<div class="button-row">

<button id="resetButton" type="button">Reset parameters</button>

<button id="exportButton" type="button">Download report snippet</button>

</div>

</section>

</aside>

<section class="plots">

<div class="panel plot-card plot-card--wide">

<div class="plot-card__header">

<div>

<h2>Outer, Middle, and Combined Response</h2>

<p>Outer-ear transfer cascaded with the middle-ear transmission model.</p>

</div>

<div class="summary-chip" id="summaryChip">Calculating...</div>

</div>

<svg

id="combinedPlot"

class="plot"

viewBox="0 0 760 280"

role="img"

aria-label="Combined frequency response plot"

></svg>

</div>

<div class="plot-grid middle-plot-grid">

<div class="panel plot-card">

<div class="plot-card__header plot-card__header--compact">

<div>

<h3>Middle-Ear Transmission</h3>

<p>Magnitude from the eardrum input to the cochlear load.</p>

</div>

</div>

<svg

id="middlePlot"

class="plot plot--small"

viewBox="0 0 360 220"

role="img"

aria-label="Middle-ear transmission magnitude plot"

></svg>

</div>

<div class="panel plot-card">

<div class="plot-card__header plot-card__header--compact">

<div>

<h3>Middle-Ear Transmission Phase</h3>

<p>Phase of the modeled pressure transfer function.</p>

</div>

</div>

<svg

id="middlePhasePlot"

class="plot plot--small"

viewBox="0 0 360 220"

role="img"

aria-label="Middle-ear transmission phase plot"

></svg>

</div>

<div class="panel plot-card">

<div class="plot-card__header plot-card__header--compact">

<div>

<h3>Middle-Ear Input Impedance</h3>

<p>Magnitude of the input impedance in model units.</p>

</div>

</div>

<svg

id="middleImpedancePlot"

class="plot plot--small"

viewBox="0 0 360 220"

role="img"

aria-label="Middle-ear input impedance plot"

></svg>

</div>

</div>

<div class="plot-grid">

<div class="panel plot-card">

<div class="plot-card__header plot-card__header--compact">

<div>

<h3>Concha Reflector</h3>

<p>Parabolic gain rise above the break frequency.</p>

</div>

</div>

<svg

id="reflectorPlot"

class="plot plot--small"

viewBox="0 0 360 220"

role="img"

aria-label="Concha reflector response plot"

></svg>

</div>

<div class="panel plot-card">

<div class="plot-card__header plot-card__header--compact">

<div>

<h3>Concha Cavity</h3>

<p>Helmholtz-like resonance from volume and opening.</p>

</div>

</div>

<svg

id="conchaPlot"

class="plot plot--small"

viewBox="0 0 360 220"

role="img"

aria-label="Concha cavity response plot"

></svg>

</div>

<div class="panel plot-card">

<div class="plot-card__header plot-card__header--compact">

<div>

<h3>Ear Canal</h3>

<p>Quarter-wave resonances with optional IEC 318 branches.</p>

</div>

</div>

<svg

id="canalPlot"

class="plot plot--small"

viewBox="0 0 360 220"

role="img"

aria-label="Ear canal response plot"

></svg>

</div>

<div class="panel plot-card">

<div class="plot-card__header plot-card__header--compact">

<div>

<h3>Pinna Reflection</h3>

<p>Comb-style interference from delayed reflected sound.</p>

</div>

</div>

<svg

id="reflectionPlot"

class="plot plot--small"

viewBox="0 0 360 220"

role="img"

aria-label="Pinna reflection response plot"

></svg>

</div>

</div>

<div class="panel plot-card plot-card--wide">

<div class="plot-card__header sweep-header">

<div>

<h2>Real-Time Parameter Sweep</h2>

<p>Overlay combined responses across the selected parameter range.</p>

</div>

<div class="sweep-controls">

<label class="select-control">

<span>Swept parameter</span>

<select id="sweepParameter"></select>

</label>

<label class="select-control">

<span>Trace count</span>

<select id="sweepTraceCount">

<option value="4">4</option>

<option value="5">5</option>

<option value="6">6</option>

<option value="7" selected>7</option>

<option value="8">8</option>

<option value="9">9</option>

</select>

</label>

</div>

</div>

<svg

id="sweepPlot"

class="plot"

viewBox="0 0 760 280"

role="img"

aria-label="Parameter sweep plot"

></svg>

</div>

<div class="results-grid">

<div class="panel result-card">

<h2>Calculated Markers</h2>

<div id="markerList" class="marker-list"></div>

</div>

<div class="panel result-card">

<h2>AI-Assisted Experiments</h2>

<p>Each experiment changes one required parameter by +20% from the current baseline.</p>

<div id="experimentList" class="experiment-list"></div>

</div>

<div class="panel result-card">

<h2>Model Equations</h2>

<div class="equation-list">

<p><code>f_break = c / (16a)</code></p>

<p><code>f_H = c / (2pi) * sqrt(A / (V L_eff))</code></p>

<p><code>f_n = n c / (4 L_eff)</code></p>

<p><code>f_notch = c / (2 r_eff)</code></p>

<p><code>Z_R = R, Z_L = jωL, Z_C = 1/(jωC)</code></p>

<p><code>H_total = H_outer × H_middle</code></p>

</div>

<p class="microcopy">

These equations follow the class notes and use a simplified

damping model so the plots stay finite and interactive.

</p>

</div>

<div class="panel result-card">

<h2>Audio Preview</h2>

<div class="audio-controls">

<div class="button-row">

<button id="playNoiseButton" type="button">Play noise</button>

<button id="playSweepButton" type="button">Play sweep</button>

<button id="stopAudioButton" type="button">Stop</button>

</div>

<label class="file-control">

<span>Upload audio</span>

<input id="audioFileInput" type="file" accept="audio/*" />

</label>

<div class="button-row">

<button id="playFileButton" type="button" disabled>

Play uploaded audio

</button>

</div>

</div>

<p class="microcopy">

Audio uses a lightweight Web Audio approximation of the plotted

model so you can hear the current anatomy settings.

</p>

<div id="audioStatus" class="audio-status">Audio idle.</div>

</div>

</div>

</section>

</main>

</div>

<script src="app.js?v=middle-ear-1"></script>

</body>

</html>

## styles.css

:root {

--bg: #f6efe3;

--bg-strong: #ead8c0;

--panel: rgba(255, 250, 242, 0.86);

--panel-strong: rgba(255, 249, 240, 0.95);

--line: rgba(94, 66, 43, 0.16);

--ink: #2d2016;

--muted: #715644;

--accent: #d5673f;

--accent-deep: #b44c2d;

--accent-soft: rgba(213, 103, 63, 0.16);

--teal: #2a7f7a;

--teal-soft: rgba(42, 127, 122, 0.18);

--gold: #c4902f;

--plot-grid: rgba(45, 32, 22, 0.12);

--shadow: 0 18px 50px rgba(73, 47, 27, 0.12);

--radius-lg: 28px;

--radius-md: 18px;

--radius-sm: 12px;

--font-body: "Avenir Next", "Trebuchet MS", "Segoe UI", sans-serif;

--font-display: "Iowan Old Style", "Palatino Linotype", "Book Antiqua", serif;

}

* {

box-sizing: border-box;

}

body {

margin: 0;

min-height: 100vh;

font-family: var(--font-body);

color: var(--ink);

background:

radial-gradient(circle at top left, rgba(212, 131, 80, 0.26), transparent 34%),

radial-gradient(circle at top right, rgba(42, 127, 122, 0.18), transparent 28%),

linear-gradient(160deg, #f9f4ed 0%, var(--bg) 52%, var(--bg-strong) 100%);

}

body::before,

body::after {

content: "";

position: fixed;

inset: auto;

width: 28rem;

height: 28rem;

border-radius: 999px;

filter: blur(24px);

opacity: 0.4;

pointer-events: none;

}

body::before {

top: -9rem;

left: -8rem;

background: radial-gradient(circle, rgba(213, 103, 63, 0.22), transparent 68%);

}

body::after {

right: -10rem;

bottom: -9rem;

background: radial-gradient(circle, rgba(42, 127, 122, 0.18), transparent 70%);

}

.page-shell {

width: min(1440px, calc(100vw - 2rem));

margin: 1rem auto 2.5rem;

position: relative;

}

.panel {

background: var(--panel);

border: 1px solid rgba(255, 255, 255, 0.68);

border-radius: var(--radius-lg);

box-shadow: var(--shadow);

backdrop-filter: blur(18px);

}

.hero {

display: grid;

grid-template-columns: minmax(0, 1.6fr) minmax(280px, 0.9fr);

gap: 1.5rem;

padding: 2rem;

overflow: hidden;

position: relative;

}

.hero::after {

content: "";

position: absolute;

right: 2rem;

top: 1.75rem;

width: 11rem;

height: 15rem;

border-radius: 58% 42% 52% 48% / 42% 44% 56% 58%;

background:

radial-gradient(circle at 34% 34%, rgba(255, 255, 255, 0.9), transparent 24%),

linear-gradient(160deg, rgba(213, 103, 63, 0.12), rgba(42, 127, 122, 0.16));

border: 1px solid rgba(113, 86, 68, 0.12);

}

.eyebrow {

margin: 0 0 0.65rem;

letter-spacing: 0.18em;

text-transform: uppercase;

font-size: 0.75rem;

color: var(--accent-deep);

font-weight: 700;

}

.hero h1 {

margin: 0;

font-family: var(--font-display);

font-size: clamp(2.2rem, 3.8vw, 4rem);

line-height: 0.95;

max-width: 13ch;

}

.hero__lede {

max-width: 58ch;

margin: 1rem 0 0;

color: var(--muted);

font-size: 1rem;

line-height: 1.65;

}

.hero__stats {

display: grid;

align-content: end;

gap: 0.85rem;

position: relative;

z-index: 1;

}

.stat-card {

background: var(--panel-strong);

border: 1px solid rgba(113, 86, 68, 0.1);

border-radius: var(--radius-md);

padding: 1rem 1.1rem;

}

.stat-card__label {

display: block;

font-size: 0.78rem;

text-transform: uppercase;

letter-spacing: 0.12em;

color: var(--muted);

margin-bottom: 0.35rem;

}

.stat-card strong {

font-size: 1rem;

}

.layout {

display: grid;

grid-template-columns: minmax(320px, 360px) minmax(0, 1fr);

gap: 1rem;

margin-top: 1rem;

align-items: start;

}

.controls {

padding: 1.25rem;

position: sticky;

top: 1rem;

}

.section-heading h2,

.plot-card h2,

.result-card h2,

.control-group h3,

.plot-card h3 {

margin: 0;

font-family: var(--font-display);

font-weight: 700;

}

.section-heading p,

.control-group__heading p,

.plot-card p,

.result-card p {

margin: 0.35rem 0 0;

color: var(--muted);

line-height: 1.55;

}

.control-group {

margin-top: 1.2rem;

padding: 1rem;

background: rgba(255, 255, 255, 0.48);

border-radius: var(--radius-md);

border: 1px solid rgba(113, 86, 68, 0.08);

}

.control-group--middle {

background:

linear-gradient(145deg, rgba(139, 91, 157, 0.1), rgba(255, 255, 255, 0.48));

border-color: rgba(139, 91, 157, 0.2);

}

.topology-note {

padding-top: 0.65rem;

border-top: 1px solid rgba(113, 86, 68, 0.12);

}

.control-group__heading {

margin-bottom: 0.8rem;

}

.control + .control,

.toggle + .control,

.control + .toggle {

margin-top: 0.9rem;

}

.control {

display: block;

}

.control__top {

display: flex;

justify-content: space-between;

gap: 0.75rem;

font-size: 0.95rem;

margin-bottom: 0.45rem;

}

.control output {

font-weight: 700;

color: var(--accent-deep);

}

input[type="range"] {

width: 100%;

margin: 0;

accent-color: var(--accent);

}

.toggle {

display: flex;

gap: 0.75rem;

align-items: start;

font-size: 0.93rem;

color: var(--ink);

}

.toggle input {

margin-top: 0.1rem;

accent-color: var(--teal);

}

.button-row {

display: flex;

flex-wrap: wrap;

gap: 0.7rem;

}

button,

select,

input[type="file"] {

font: inherit;

}

button {

border: 0;

border-radius: 999px;

padding: 0.78rem 1.1rem;

background: linear-gradient(135deg, var(--accent), var(--accent-deep));

color: white;

font-weight: 700;

cursor: pointer;

box-shadow: 0 10px 24px rgba(180, 76, 45, 0.18);

transition: transform 150ms ease, box-shadow 150ms ease, opacity 150ms ease;

}

button:hover {

transform: translateY(-1px);

box-shadow: 0 14px 28px rgba(180, 76, 45, 0.22);

}

button:disabled {

cursor: not-allowed;

opacity: 0.45;

box-shadow: none;

}

.plots {

display: grid;

gap: 1rem;

}

.plot-card {

padding: 1rem;

}

.plot-card--wide {

padding: 1.1rem 1.15rem 0.95rem;

}

.plot-card__header {

display: flex;

justify-content: space-between;

gap: 1rem;

align-items: start;

margin-bottom: 0.8rem;

}

.plot-card__header--compact {

margin-bottom: 0.55rem;

}

.summary-chip {

padding: 0.6rem 0.85rem;

border-radius: 999px;

background: var(--accent-soft);

color: var(--accent-deep);

font-weight: 700;

font-size: 0.88rem;

align-self: center;

}

.plot-grid {

display: grid;

grid-template-columns: repeat(2, minmax(0, 1fr));

gap: 1rem;

}

.middle-plot-grid {

grid-template-columns: repeat(3, minmax(0, 1fr));

}

.plot {

width: 100%;

display: block;

border-radius: 16px;

background:

linear-gradient(180deg, rgba(255, 255, 255, 0.66), rgba(255, 255, 255, 0.18)),

linear-gradient(180deg, rgba(42, 127, 122, 0.03), rgba(213, 103, 63, 0.04));

}

.plot--small {

min-height: 220px;

}

.sweep-header {

align-items: end;

}

.sweep-controls {

display: flex;

flex-wrap: wrap;

gap: 0.85rem;

}

.select-control {

display: grid;

gap: 0.35rem;

min-width: 170px;

font-size: 0.88rem;

}

.select-control span {

color: var(--muted);

}

select,

.file-control input[type="file"] {

border: 1px solid rgba(113, 86, 68, 0.18);

border-radius: 12px;

background: rgba(255, 255, 255, 0.9);

padding: 0.7rem 0.8rem;

}

.results-grid {

display: grid;

grid-template-columns: repeat(3, minmax(0, 1fr));

gap: 1rem;

}

.result-card {

padding: 1.1rem;

}

.marker-list {

display: grid;

gap: 0.7rem;

margin-top: 0.9rem;

}

.marker-row {

display: flex;

justify-content: space-between;

gap: 0.75rem;

padding: 0.78rem 0.9rem;

border-radius: 14px;

background: rgba(255, 255, 255, 0.55);

border: 1px solid rgba(113, 86, 68, 0.08);

}

.marker-row strong {

font-size: 0.95rem;

}

.marker-row span:last-child {

font-weight: 700;

color: var(--teal);

}

.experiment-list {

display: grid;

gap: 0.75rem;

margin-top: 0.9rem;

}

.experiment-row {

padding: 0.85rem 0.9rem;

border-radius: 14px;

background: rgba(139, 91, 157, 0.08);

border: 1px solid rgba(139, 91, 157, 0.14);

}

.experiment-row strong {

color: #6f427f;

}

.experiment-row p {

margin: 0.45rem 0 0;

font-size: 0.86rem;

line-height: 1.45;

}

.experiment-row p span {

font-weight: 700;

}

.equation-list {

display: grid;

gap: 0.55rem;

margin-top: 0.95rem;

}

.equation-list p {

margin: 0;

padding: 0.75rem 0.85rem;

border-radius: 14px;

background: var(--teal-soft);

color: var(--ink);

}

code {

font-family: "SFMono-Regular", "Menlo", "Consolas", monospace;

font-size: 0.92rem;

}

.microcopy {

font-size: 0.9rem;

}

.audio-controls {

display: grid;

gap: 0.8rem;

margin-top: 0.9rem;

}

.file-control {

display: grid;

gap: 0.35rem;

font-size: 0.88rem;

color: var(--muted);

}

.audio-status {

margin-top: 0.75rem;

padding: 0.85rem 0.95rem;

border-radius: 14px;

background: rgba(42, 127, 122, 0.08);

color: var(--teal);

font-weight: 700;

}

.axis-label,

.tick-label,

.legend-label,

.marker-label {

font-family: var(--font-body);

fill: var(--muted);

}

.axis-label {

font-size: 12px;

font-weight: 700;

}

.tick-label {

font-size: 11px;

}

.legend-label {

font-size: 11px;

font-weight: 700;

}

.marker-label {

font-size: 11px;

}

@media (max-width: 1120px) {

.layout {

grid-template-columns: 1fr;

}

.controls {

position: static;

}

.results-grid {

grid-template-columns: 1fr;

}

}

@media (max-width: 820px) {

.page-shell {

width: min(100vw, calc(100vw - 1rem));

margin: 0.5rem auto 2rem;

}

.hero {

grid-template-columns: 1fr;

padding: 1.4rem;

}

.hero::after {

width: 8.2rem;

height: 11rem;

right: 1rem;

top: 1rem;

}

.plot-grid {

grid-template-columns: 1fr;

}

.middle-plot-grid {

grid-template-columns: 1fr;

}

.plot-card__header,

.sweep-header {

flex-direction: column;

}

.sweep-controls {

width: 100%;

}

.select-control {

min-width: 0;

width: 100%;

}

}

## app.js

const SPEED_OF_SOUND = 343;

const ANALYSIS_SAMPLE_RATE = 96000;

const FREQUENCIES = logSpace(20, 20000, 480);

const DB_RANGE = { min: -18, max: 24 };

const PARAMETER_DEFS = [

{

key: "canalLengthMm",

label: "Canal length",

unit: "mm",

min: 20,

max: 40,

step: 0.5,

defaultValue: 27,

},

{

key: "canalDiameterMm",

label: "Canal diameter",

unit: "mm",

min: 4,

max: 10,

step: 0.1,

defaultValue: 7,

},

{

key: "conchaDepthMm",

label: "Concha depth",

unit: "mm",

min: 5,

max: 20,

step: 0.5,

defaultValue: 12,

},

{

key: "conchaVolumeCc",

label: "Concha volume",

unit: "cc",

min: 1,

max: 8,

step: 0.1,

defaultValue: 3,

},

{

key: "conchaApertureMm",

label: "Aperture radius",

unit: "mm",

min: 2,

max: 8,

step: 0.1,

defaultValue: 4.5,

},

{

key: "reflectorLengthMm",

label: "Reflector length L",

unit: "mm",

min: 2,

max: 12,

step: 0.2,

defaultValue: 5,

},

{

key: "reflectorRadiusMm",

label: "Reflector radius a",

unit: "mm",

min: 5,

max: 18,

step: 0.2,

defaultValue: 10,

},

{

key: "reflectionPathMm",

label: "Base reflection path",

unit: "mm",

min: 8,

max: 24,

step: 0.2,

defaultValue: 14,

},

{

key: "reflectionStrength",

label: "Reflection strength",

unit: "",

min: 0.05,

max: 0.95,

step: 0.01,

defaultValue: 0.6,

},

{

key: "azimuthDeg",

label: "Source azimuth",

unit: "deg",

min: -90,

max: 90,

step: 1,

defaultValue: 0,

},

{

key: "elevationDeg",

label: "Source elevation",

unit: "deg",

min: -45,

max: 90,

step: 1,

defaultValue: 0,

},

{

key: "middleInertance",

label: "Eardrum + ossicle inertance",

unit: "m.u.",

min: 0.0004,

max: 0.003,

step: 0.0001,

defaultValue: 0.0012,

},

{

key: "middleCompliance",

label: "Eardrum compliance",

unit: "m.u.",

min: 0.000008,

max: 0.00006,

step: 0.000001,

defaultValue: 0.00002,

},

{

key: "middleResistance",

label: "Eardrum/joint resistance",

unit: "m.u.",

min: 0.1,

max: 2,

step: 0.05,

defaultValue: 0.65,

},

{

key: "middleCavityVolumeCc",

label: "Middle-ear cavity volume",

unit: "cc",

min: 0.5,

max: 3,

step: 0.1,

defaultValue: 1.5,

},

{

key: "jointCompliance",

label: "Incudo-stapedial compliance",

unit: "m.u.",

min: 0.000012,

max: 0.00008,

step: 0.000001,

defaultValue: 0.000035,

},

{

key: "cochlearLoad",

label: "Cochlear load resistance",

unit: "m.u.",

min: 0.5,

max: 4,

step: 0.1,

defaultValue: 1.8,

},

];

const PARAMETER_LOOKUP = Object.fromEntries(

PARAMETER_DEFS.map((definition) => [definition.key, definition]),

);

const DEFAULTS = Object.fromEntries(

PARAMETER_DEFS.map((definition) => [definition.key, definition.defaultValue]),

);

const SWEEPABLE_KEYS = [

"canalLengthMm",

"canalDiameterMm",

"conchaDepthMm",

"conchaVolumeCc",

"conchaApertureMm",

"reflectorLengthMm",

"reflectorRadiusMm",

"reflectionPathMm",

"reflectionStrength",

"azimuthDeg",

"elevationDeg",

"middleInertance",

"middleCompliance",

"middleResistance",

"middleCavityVolumeCc",

"jointCompliance",

"cochlearLoad",

];

const COLORS = {

outer: "#2a7f7a",

middle: "#8b5b9d",

total: "#d5673f",

combined: "#d5673f",

reflector: "#c4902f",

concha: "#2a7f7a",

canal: "#2f5f9d",

reflection: "#b44c2d",

};

const dom = {

parameterInputs: Array.from(document.querySelectorAll("[data-param]")),

iecToggle: document.getElementById("iec318Enabled"),

resetButton: document.getElementById("resetButton"),

exportButton: document.getElementById("exportButton"),

sweepParameter: document.getElementById("sweepParameter"),

sweepTraceCount: document.getElementById("sweepTraceCount"),

markerList: document.getElementById("markerList"),

summaryChip: document.getElementById("summaryChip"),

audioStatus: document.getElementById("audioStatus"),

audioFileInput: document.getElementById("audioFileInput"),

playNoiseButton: document.getElementById("playNoiseButton"),

playSweepButton: document.getElementById("playSweepButton"),

playFileButton: document.getElementById("playFileButton"),

stopAudioButton: document.getElementById("stopAudioButton"),

plots: {

combined: document.getElementById("combinedPlot"),

reflector: document.getElementById("reflectorPlot"),

concha: document.getElementById("conchaPlot"),

canal: document.getElementById("canalPlot"),

reflection: document.getElementById("reflectionPlot"),

middle: document.getElementById("middlePlot"),

middlePhase: document.getElementById("middlePhasePlot"),

middleImpedance: document.getElementById("middleImpedancePlot"),

sweep: document.getElementById("sweepPlot"),

},

experimentList: document.getElementById("experimentList"),

};

const audioState = {

context: null,

source: null,

sourceNodes: [],

activeNodes: [],

uploadedBuffer: null,

};

let latestModel = null;

initialize();

function initialize() {

buildSweepMenu();

bindParameterEvents();

bindActionEvents();

refreshParameterReadouts();

updateView();

}

function buildSweepMenu() {

SWEEPABLE_KEYS.forEach((key) => {

const option = document.createElement("option");

option.value = key;

option.textContent = PARAMETER_LOOKUP[key].label;

dom.sweepParameter.appendChild(option);

});

dom.sweepParameter.value = "canalLengthMm";

}

function bindParameterEvents() {

dom.parameterInputs.forEach((input) => {

input.addEventListener("input", () => {

updateReadout(input.dataset.param, Number(input.value));

updateView();

});

});

dom.iecToggle.addEventListener("change", updateView);

dom.sweepParameter.addEventListener("change", updateView);

dom.sweepTraceCount.addEventListener("change", updateView);

}

function bindActionEvents() {

dom.resetButton.addEventListener("click", resetParameters);

dom.exportButton.addEventListener("click", exportReportSnippet);

dom.playNoiseButton.addEventListener("click", playNoisePreview);

dom.playSweepButton.addEventListener("click", playToneSweepPreview);

dom.stopAudioButton.addEventListener("click", stopAudio);

dom.playFileButton.addEventListener("click", playUploadedAudio);

dom.audioFileInput.addEventListener("change", handleAudioUpload);

}

function refreshParameterReadouts() {

PARAMETER_DEFS.forEach((definition) => {

const input = getInput(definition.key);

updateReadout(definition.key, Number(input.value));

});

}

function resetParameters() {

PARAMETER_DEFS.forEach((definition) => {

const input = getInput(definition.key);

input.value = definition.defaultValue;

});

dom.iecToggle.checked = true;

refreshParameterReadouts();

updateView();

setAudioStatus("Parameters reset to the baseline anatomy.");

}

function updateReadout(key, value) {

const output = document.getElementById(`${key}Value`);

if (output) {

output.textContent = formatParameterValue(key, value);

}

}

function readParameters() {

const parameters = {};

PARAMETER_DEFS.forEach((definition) => {

parameters[definition.key] = Number(getInput(definition.key).value);

});

parameters.iec318Enabled = dom.iecToggle.checked;

return parameters;

}

function updateView() {

const parameters = readParameters();

const model = computeModel(parameters);

latestModel = model;

renderPlot(dom.plots.combined, [

{ label: "Outer ear", values: model.outerDb, color: COLORS.outer },

{ label: "Middle ear", values: model.middleDb, color: COLORS.middle },

{ label: "Combined", values: model.combinedTotalDb, color: COLORS.total },

], {

yRange: DB_RANGE,

markers: [

{

frequency: model.metrics.reflector.breakHz,

label: "Reflector break",

},

{

frequency: model.metrics.concha.helmholtzHz,

label: "Helmholtz",

},

{

frequency: model.metrics.canal.f1,

label: "Canal f1",

},

{

frequency: model.metrics.reflection.notchHz,

label: "Pinna notch",

},

],

});

renderPlot(dom.plots.reflector, [

{ label: "Reflector", values: model.reflectorDb, color: COLORS.reflector },

], { yRange: DB_RANGE });

renderPlot(dom.plots.concha, [

{ label: "Concha", values: model.conchaDb, color: COLORS.concha },

], { yRange: DB_RANGE });

renderPlot(dom.plots.canal, [

{ label: "Canal", values: model.canalDb, color: COLORS.canal },

], { yRange: DB_RANGE });

renderPlot(dom.plots.reflection, [

{

label: "Reflection",

values: model.reflectionDb,

color: COLORS.reflection,

},

], { yRange: DB_RANGE });

renderPlot(dom.plots.middle, [

{ label: "Middle ear", values: model.middleDb, color: COLORS.middle },

], {

yRange: { min: -36, max: 12 },

yTicks: [12, 6, 0, -6, -12, -18, -24, -30, -36],

markers: [

{

frequency: model.metrics.middle.transmissionPeak.frequency,

label: "Transmission peak",

},

],

});

renderPlot(dom.plots.middlePhase, [

{ label: "Phase", values: model.middlePhaseDeg, color: COLORS.middle },

], {

yRange: { min: -180, max: 180 },

yTicks: [-180, -120, -60, 0, 60, 120, 180],

});

renderPlot(dom.plots.middleImpedance, [

{

label: "Input impedance",

values: model.middleInputImpedanceDb,

color: COLORS.canal,

},

], {

yRange: { min: -12, max: 30 },

yTicks: [30, 24, 18, 12, 6, 0, -6, -12],

});

renderPlot(dom.plots.sweep, buildSweepSeries(parameters), {

yRange: DB_RANGE,

showLegend: true,

});

renderMarkerList(model);

renderExperiments(parameters, model);

renderSummaryChip(model);

}

function computeModel(parameters) {

const reflector = computeReflectorResponse(parameters);

const concha = computeConchaResponse(parameters);

const canal = computeCanalResponse(parameters);

const reflection = computeReflectionResponse(parameters);

const middle = computeMiddleEarResponse(parameters);

const combinedMagnitude = FREQUENCIES.map((_, index) => (

reflector.magnitudes[index] *

concha.magnitudes[index] *

canal.magnitudes[index] *

reflection.magnitudes[index]

));

const outerDb = combinedMagnitude.map(magnitudeToDb);

const combinedTotalMagnitude = combinedMagnitude.map(

(magnitude, index) => magnitude * middle.magnitudes[index],

);

const combinedTotalDb = combinedTotalMagnitude.map(magnitudeToDb);

const combinedPeak = findExtremum(FREQUENCIES, combinedTotalDb, "max");

const highBandNotch = findExtremum(

FREQUENCIES.filter((frequency) => frequency >= 4000),

combinedTotalDb.slice(FREQUENCIES.findIndex((frequency) => frequency >= 4000)),

"min",

);

return {

parameters,

reflectorDb: reflector.magnitudes.map(magnitudeToDb),

conchaDb: concha.magnitudes.map(magnitudeToDb),

canalDb: canal.magnitudes.map(magnitudeToDb),

reflectionDb: reflection.magnitudes.map(magnitudeToDb),

combinedDb: combinedTotalDb,

outerDb,

middleDb: middle.magnitudes.map(magnitudeToDb),

middlePhaseDeg: middle.phasesDeg,

middleInputImpedanceDb: middle.inputImpedanceMagnitudes.map(magnitudeToDb),

combinedTotalDb,

metrics: {

reflector: reflector.metrics,

concha: concha.metrics,

canal: canal.metrics,

reflection: reflection.metrics,

middle: middle.metrics,

combinedPeak,

highBandNotch,

},

};

}

function computeReflectorResponse(parameters) {

const reflectorLength = millimetersToMeters(parameters.reflectorLengthMm);

const reflectorRadius = millimetersToMeters(parameters.reflectorRadiusMm);

const ratio = reflectorLength / reflectorRadius;

const breakHz = SPEED_OF_SOUND / (16 * reflectorRadius);

const slopeDbPerOctave = clamp(6 * (ratio / 0.5), 3.5, 9.0);

const maxGainDb = clamp(12 * Math.sqrt(ratio / 0.5), 6, 18);

const magnitudes = FREQUENCIES.map((frequency) => {

const octavesAboveBreak = Math.max(0, Math.log2(frequency / breakHz));

const gainDb = Math.min(maxGainDb, slopeDbPerOctave * octavesAboveBreak);

return dbToMagnitude(gainDb);

});

return {

magnitudes,

metrics: {

breakHz,

ratio,

slopeDbPerOctave,

maxGainDb,

},

};

}

function computeConchaResponse(parameters) {

const volume = cubicCentimetersToCubicMeters(parameters.conchaVolumeCc);

const radius = millimetersToMeters(parameters.conchaApertureMm);

const apertureArea = Math.PI * radius ** 2;

const depth = millimetersToMeters(parameters.conchaDepthMm);

const effectiveLength = depth + 1.7 * radius;

const helmholtzHz =

(SPEED_OF_SOUND / (2 * Math.PI)) *

Math.sqrt(apertureArea / (volume * effectiveLength));

const resonanceIndex = Math.sqrt(apertureArea / (volume * effectiveLength));

const q = clamp(0.7 + 2.2 * (radius / Math.max(depth, 0.001)), 0.7, 3.1);

const gainDb = clamp(5 + 4 * Math.log2(resonanceIndex / 32), 2, 12);

const filter = makePeakingFilter(helmholtzHz, q, gainDb);

const magnitudes = FREQUENCIES.map((frequency) =>

biquadMagnitudeAtFrequency(filter, frequency),

);

return {

magnitudes,

metrics: {

helmholtzHz,

effectiveLength,

apertureArea,

q,

gainDb,

},

};

}

function computeCanalResponse(parameters) {

const radius = millimetersToMeters(parameters.canalDiameterMm / 2);

const length = millimetersToMeters(parameters.canalLengthMm);

const effectiveLength = length + 0.6 * radius;

const f1 = SPEED_OF_SOUND / (4 * effectiveLength);

const f2 = 2 * f1;

const f3 = 3 * f1;

const narrowness = clamp(Math.sqrt(7 / parameters.canalDiameterMm), 0.7, 1.55);

const q1 = clamp(1.25 * narrowness, 0.85, 2.3);

const q2 = clamp(1.05 * narrowness, 0.8, 2.1);

const q3 = clamp(1.7 * narrowness, 1.0, 2.8);

const gain1Db = clamp(6.5 * narrowness, 4, 11);

const notchDepthDb = clamp(4.3 * narrowness, 2.5, 8);

const gain3Db = clamp(3.2 * narrowness, 1.5, 6.5);

const filters = [

makePeakingFilter(f1, q1, gain1Db),

makeNotchFilter(f2, q2, notchDepthDb),

makePeakingFilter(f3, q3, gain3Db),

];

if (parameters.iec318Enabled) {

filters.push(makePeakingFilter(220, 0.85, 1.4));

filters.push(makePeakingFilter(2000, 1.05, 2.3));

}

const magnitudes = FREQUENCIES.map((frequency) =>

filters.reduce(

(product, filter) => product * biquadMagnitudeAtFrequency(filter, frequency),

1,

),

);

return {

magnitudes,

metrics: {

effectiveLength,

f1,

f2,

f3,

q1,

q2,

q3,

gain1Db,

notchDepthDb,

gain3Db,

},

};

}

function computeReflectionResponse(parameters) {

const direction = deriveDirection(parameters);

const effectivePathMeters = millimetersToMeters(direction.effectivePathMm);

const effectiveStrength = direction.effectiveStrength;

const absorptionCornerHz = 12000;

const notchHz = SPEED_OF_SOUND / (2 * effectivePathMeters);

const magnitudes = FREQUENCIES.map((frequency) => {

const attenuation =

effectiveStrength / Math.sqrt(1 + (frequency / absorptionCornerHz) ** 2);

const phase = (2 * Math.PI * frequency * effectivePathMeters) / SPEED_OF_SOUND;

const real = 1 + attenuation * Math.cos(phase);

const imaginary = -attenuation * Math.sin(phase);

return Math.sqrt(real ** 2 + imaginary ** 2);

});

return {

magnitudes,

metrics: {

effectivePathMm: direction.effectivePathMm,

effectiveStrength,

notchHz,

},

};

}

function deriveDirection(parameters) {

const azimuth = degreesToRadians(parameters.azimuthDeg);

const elevation = degreesToRadians(parameters.elevationDeg);

const directionScale =

1 +

0.22 * Math.sin(azimuth) +

0.18 * Math.sin(elevation) -

0.08 * (1 - Math.cos(azimuth) * Math.cos(elevation));

const effectivePathMm = clamp(parameters.reflectionPathMm * directionScale, 8, 24);

const effectiveStrength = clamp(

parameters.reflectionStrength *

(1 +

0.22 * Math.abs(Math.sin(azimuth)) +

0.12 * Math.max(0, Math.sin(elevation))),

0.05,

0.98,

);

return { effectivePathMm, effectiveStrength };

}

// The middle-ear network is intentionally normalized rather than clinically calibrated.

// It preserves the frequency trends of R, L, and C elements while keeping the controls stable.

function computeMiddleEarResponse(parameters) {

const seriesImpedances = FREQUENCIES.map((frequency) => {

const eardrum = Z_eardrum(frequency, parameters);

const ossicles = Z_ossicles(frequency, parameters);

const joint = Z_joint(frequency, parameters);

const cochlea = Z_cochlea(frequency, parameters);

const seriesPath = addComplex(addComplex(eardrum, ossicles), addComplex(joint, cochlea));

const cavity = Z_cavity(frequency, parameters);

const input = parallelImpedance(seriesPath, cavity);

const source = complex(4.0, 0);

const inputDivider = divideComplex(input, addComplex(source, input));

const loadTransfer = divideComplex(cochlea, seriesPath);

const transmission = scaleComplex(multiplyComplex(inputDivider, loadTransfer), 4.5);

return { input, transmission };

});

const magnitudes = seriesImpedances.map(({ transmission }) => complexMagnitude(transmission));

const phasesDeg = seriesImpedances.map(({ transmission }) =>

(Math.atan2(transmission.imaginary, transmission.real) * 180) / Math.PI,

);

const inputImpedanceMagnitudes = seriesImpedances.map(({ input }) => complexMagnitude(input));

const transmissionDb = magnitudes.map(magnitudeToDb);

const inputImpedanceDb = inputImpedanceMagnitudes.map(magnitudeToDb);

return {

magnitudes,

phasesDeg,

inputImpedanceMagnitudes,

metrics: {

transmissionPeak: findExtremum(FREQUENCIES, transmissionDb, "max"),

inputImpedancePeak: findExtremum(FREQUENCIES, inputImpedanceDb, "max"),

},

};

}

function Z_cavity(frequency, parameters) {

const omega = 2 * Math.PI * frequency;

const cavityCompliance = parameters.middleCavityVolumeCc * 0.000002;

return addComplex(

complex(0.12, 0),

impedanceCompliance(omega, cavityCompliance),

);

}

function Z_eardrum(frequency, parameters) {

const omega = 2 * Math.PI * frequency;

return addComplex(

complex(parameters.middleResistance * 0.82, 0),

addComplex(

impedanceInertance(omega, parameters.middleInertance * 0.75),

impedanceCompliance(omega, parameters.middleCompliance),

),

);

}

function Z_ossicles(frequency, parameters) {

const omega = 2 * Math.PI * frequency;

return impedanceInertance(omega, parameters.middleInertance * 0.25);

}

function Z_joint(frequency, parameters) {

const omega = 2 * Math.PI * frequency;

return addComplex(

complex(parameters.middleResistance * 0.18, 0),

impedanceCompliance(omega, parameters.jointCompliance),

);

}

function Z_cochlea(frequency, parameters) {

const omega = 2 * Math.PI * frequency;

return addComplex(

complex(parameters.cochlearLoad, 0),

impedanceInertance(omega, 0.00006),

);

}

function impedanceInertance(omega, inertance) {

return complex(0, omega * inertance);

}

function impedanceCompliance(omega, compliance) {

return complex(0, -1 / Math.max(omega * compliance, 1e-12));

}

function complex(real, imaginary) {

return { real, imaginary };

}

function addComplex(first, second) {

return complex(first.real + second.real, first.imaginary + second.imaginary);

}

function multiplyComplex(first, second) {

return complex(

first.real * second.real - first.imaginary * second.imaginary,

first.real * second.imaginary + first.imaginary * second.real,

);

}

function divideComplex(numerator, denominator) {

const denominatorMagnitudeSquared =

denominator.real ** 2 + denominator.imaginary ** 2;

return complex(

(numerator.real * denominator.real + numerator.imaginary * denominator.imaginary) /

denominatorMagnitudeSquared,

(numerator.imaginary * denominator.real - numerator.real * denominator.imaginary) /

denominatorMagnitudeSquared,

);

}

function scaleComplex(value, scalar) {

return complex(value.real * scalar, value.imaginary * scalar);

}

function complexMagnitude(value) {

return Math.sqrt(value.real ** 2 + value.imaginary ** 2);

}

function parallelImpedance(first, second) {

return divideComplex(

multiplyComplex(first, second),

addComplex(first, second),

);

}

function buildSweepSeries(parameters) {

const key = dom.sweepParameter.value;

const traceCount = Number(dom.sweepTraceCount.value);

const definition = PARAMETER_LOOKUP[key];

const series = [];

for (let index = 0; index < traceCount; index += 1) {

const ratio = traceCount === 1 ? 0 : index / (traceCount - 1);

const sweptValue = interpolate(definition.min, definition.max, ratio);

const sweptParameters = { ...parameters, [key]: sweptValue };

const model = computeModel(sweptParameters);

const hue = 190 + index * 18;

series.push({

label: `${definition.label}: ${formatParameterValue(key, sweptValue)}`,

values: model.combinedTotalDb,

color: `hsl(${hue} 58% ${index === traceCount - 1 ? "42%" : "52%"})`,

});

}

return series;

}

function renderMarkerList(model) {

const rows = [

["Reflector break", formatFrequency(model.metrics.reflector.breakHz)],

["Helmholtz resonance", formatFrequency(model.metrics.concha.helmholtzHz)],

["Canal mode n = 1", formatFrequency(model.metrics.canal.f1)],

["Canal antiresonance n = 2", formatFrequency(model.metrics.canal.f2)],

["Canal mode n = 3", formatFrequency(model.metrics.canal.f3)],

["Pinna notch", formatFrequency(model.metrics.reflection.notchHz)],

[

"Effective reflection path",

`${model.metrics.reflection.effectivePathMm.toFixed(1)} mm`,

],

[

"Combined response peak",

`${formatFrequency(model.metrics.combinedPeak.frequency)} @ ${formatDb(model.metrics.combinedPeak.value)}`,

],

[

"Middle-ear transmission peak",

`${formatFrequency(model.metrics.middle.transmissionPeak.frequency)} @ ${formatDb(model.metrics.middle.transmissionPeak.value)}`,

],

[

"Input impedance peak",

formatFrequency(model.metrics.middle.inputImpedancePeak.frequency),

],

];

dom.markerList.innerHTML = rows

.map(

([label, value]) => `

<div class="marker-row">

<strong>${label}</strong>

<span>${value}</span>

</div>

`,

)

.join("");

}

function renderExperiments(parameters, baselineModel) {

const experiments = [

{

key: "middleInertance",

label: "Inertance +20%",

prediction: "The resonance should move lower because f0 is proportional to 1/sqrt(LC).",

},

{

key: "middleCompliance",

label: "Compliance +20%",

prediction: "The resonance should move lower and the reactive balance should change near the middle-ear peak.",

},

{

key: "middleResistance",

label: "Resistance +20%",

prediction: "Loss should increase, reducing the sharpness and usually lowering the peak magnitude.",

},

];

dom.experimentList.innerHTML = experiments

.map((experiment) => {

const alteredParameters = {

...parameters,

[experiment.key]: parameters[experiment.key] * 1.2,

};

const alteredModel = computeModel(alteredParameters);

const baselinePeak = baselineModel.metrics.middle.transmissionPeak;

const alteredPeak = alteredModel.metrics.middle.transmissionPeak;

const frequencyChange = alteredPeak.frequency - baselinePeak.frequency;

const levelChange = alteredPeak.value - baselinePeak.value;

const direction = frequencyChange < -1 ? "lower" : frequencyChange > 1 ? "higher" : "nearly unchanged";

return `

<article class="experiment-row">

<strong>${experiment.label}</strong>

<p><span>Prediction:</span> ${experiment.prediction}</p>

<p><span>Model result:</span> peak moved ${direction} to ${formatFrequency(alteredPeak.frequency)} (${formatSignedHz(frequencyChange)}); peak level changed ${formatDb(levelChange)}.</p>

</article>

`;

})

.join("");

}

function renderSummaryChip(model) {

const peak = model.metrics.combinedPeak;

const notch = model.metrics.reflection;

dom.summaryChip.textContent =

`Total peak ${formatFrequency(peak.frequency)} - middle-ear peak ${formatFrequency(model.metrics.middle.transmissionPeak.frequency)} - pinna notch ${formatFrequency(notch.notchHz)}`;

}

function exportReportSnippet() {

const model = latestModel;

if (!model) {

return;

}

const parameters = model.parameters;

const lines = [

"# Outer + Middle Ear Model Summary",

"",

`Date: ${new Date().toISOString().slice(0, 10)}`,

"",

"## Current anatomical parameters",

...PARAMETER_DEFS.map(

(definition) =>

`- ${definition.label}: ${formatParameterValue(

definition.key,

parameters[definition.key],

)}`,

),

`- IEC 318 auxiliary branches: ${parameters.iec318Enabled ? "On" : "Off"}`,

"",

"## Calculated frequencies",

`- Reflector break: ${formatFrequency(model.metrics.reflector.breakHz)}`,

`- Concha Helmholtz resonance: ${formatFrequency(model.metrics.concha.helmholtzHz)}`,

`- Ear-canal quarter-wave mode (n=1): ${formatFrequency(model.metrics.canal.f1)}`,

`- Ear-canal antiresonance estimate (n=2): ${formatFrequency(model.metrics.canal.f2)}`,

`- Ear-canal third mode (n=3): ${formatFrequency(model.metrics.canal.f3)}`,

`- Pinna reflection notch: ${formatFrequency(model.metrics.reflection.notchHz)}`,

`- Effective reflection path: ${model.metrics.reflection.effectivePathMm.toFixed(1)} mm`,

`- Middle-ear transmission peak: ${formatFrequency(model.metrics.middle.transmissionPeak.frequency)} @ ${formatDb(model.metrics.middle.transmissionPeak.value)}`,

`- Middle-ear input impedance peak: ${formatFrequency(model.metrics.middle.inputImpedancePeak.frequency)}`,

"",

"## Equations used",

"- f_break = c / (16a)",

"- f_H = c / 2pi * sqrt(A / (V L_eff))",

"- f_n = n c / (4 L_eff)",

"- f_notch = c / (2 r_eff)",

"- Z_R = R",

"- Z_L = j omega L",

"- Z_C = 1 / (j omega C)",

"- Z_in = Z_series || Z_cavity",

"- H_total = H_outer x H_middle",

"",

"## Middle-ear topology assumption",

"- The cavity is represented as a shunt compliance across a series path containing the eardrum, ossicles, incudo-stapedial joint, and cochlear load.",

"- The outer and middle models are cascaded without pressure-loading feedback from the middle ear into the outer-ear filters.",

"",

"## Brief discussion",

"- Increasing ear-canal length lowers the quarter-wave resonances.",

"- Increasing concha aperture or decreasing concha volume raises the Helmholtz resonance.",

"- Larger effective reflection paths push the pinna notch downward in frequency.",

"- The combined response is the product of the outer-ear response and middle-ear transmission, so middle-ear mass/compliance/loss changes reshape the total response.",

"",

"## AI-assisted experiments",

...buildExperimentReportLines(parameters, model),

"",

"## Parameter provenance",

"- Canal length, concha dimensions, and cavity volume are physically interpretable anatomical parameters, but the baseline values are assumed population-representative values.",

"- Middle-ear inertance, compliance, resistance, and cochlear load are normalized model parameters; they are assumed/fitted placeholders rather than patient measurements.",

"",

"## Model limitations",

"- The middle-ear circuit is a simplified lumped network and must be checked against the exact class-supplied series/parallel topology.",

"- The cascade assumes no loading feedback between the middle-ear input impedance and outer-ear transfer function.",

"- The model is not subject-specific and does not represent nonlinear tissue behavior, three-dimensional radiation, or individual HRTFs.",

"",

"## Copilot note",

"- This interactive model and the summary text were built with GPT-5.6 in Codex as a coding copilot. AI helped translate the class equations into JavaScript, but the circuit topology and assumptions must be checked against the class diagram before submission.",

"",

];

const blob = new Blob([lines.join("\n")], { type: "text/markdown" });

const url = URL.createObjectURL(blob);

const link = document.createElement("a");

link.href = url;

link.download = "outer-ear-model-summary.md";

link.click();

URL.revokeObjectURL(url);

}

function buildExperimentReportLines(parameters, baselineModel) {

return [

["middleInertance", "Eardrum + ossicle inertance"],

["middleCompliance", "Eardrum compliance"],

["middleResistance", "Eardrum/joint resistance"],

].flatMap(([key, label]) => {

const alteredModel = computeModel({

...parameters,

[key]: parameters[key] * 1.2,

});

const baselinePeak = baselineModel.metrics.middle.transmissionPeak;

const alteredPeak = alteredModel.metrics.middle.transmissionPeak;

return [

`- ${label} +20%: peak changed from ${formatFrequency(baselinePeak.frequency)} to ${formatFrequency(alteredPeak.frequency)}; peak level changed ${formatDb(alteredPeak.value - baselinePeak.value)}.`,

];

});

}

function renderPlot(svg, series, options = {}) {

const [width, height] = svg

.getAttribute("viewBox")

.split(" ")

.slice(2)

.map(Number);

const padding = { top: 16, right: 18, bottom: 34, left: 54 };

const innerWidth = width - padding.left - padding.right;

const innerHeight = height - padding.top - padding.bottom;

const yRange = options.yRange || DB_RANGE;

svg.innerHTML = "";

const background = createSvgElement("rect", {

x: 0,

y: 0,

width,

height,

rx: 16,

fill: "transparent",

});

svg.appendChild(background);

const yTicks = options.yTicks || [24, 18, 12, 6, 0, -6, -12, -18];

yTicks.forEach((tick) => {

const y = padding.top + innerHeight * (1 - (tick - yRange.min) / (yRange.max - yRange.min));

svg.appendChild(

createSvgElement("line", {

x1: padding.left,

y1: y,

x2: width - padding.right,

y2: y,

stroke: "var(--plot-grid)",

"stroke-width": 1,

}),

);

const label = createSvgElement("text", {

x: padding.left - 10,

y: y + 4,

"text-anchor": "end",

class: "tick-label",

});

label.textContent = `${tick}`;

svg.appendChild(label);

});

const frequencyTicks = [20, 50, 100, 200, 500, 1000, 2000, 5000, 10000, 20000];

frequencyTicks.forEach((tick) => {

const x = padding.left + innerWidth * normalizeLogFrequency(tick);

svg.appendChild(

createSvgElement("line", {

x1: x,

y1: padding.top,

x2: x,

y2: height - padding.bottom,

stroke: "var(--plot-grid)",

"stroke-width": 1,

}),

);

const label = createSvgElement("text", {

x,

y: height - 12,

"text-anchor": "middle",

class: "tick-label",

});

label.textContent = formatFrequencyTick(tick);

svg.appendChild(label);

});

svg.appendChild(

createSvgElement("rect", {

x: padding.left,

y: padding.top,

width: innerWidth,

height: innerHeight,

fill: "none",

stroke: "rgba(45, 32, 22, 0.18)",

"stroke-width": 1.2,

rx: 12,

}),

);

if (options.markers) {

options.markers.forEach((marker, index) => {

const x = padding.left + innerWidth * normalizeLogFrequency(marker.frequency);

svg.appendChild(

createSvgElement("line", {

x1: x,

y1: padding.top,

x2: x,

y2: height - padding.bottom,

stroke: index % 2 === 0 ? "rgba(42, 127, 122, 0.45)" : "rgba(213, 103, 63, 0.45)",

"stroke-width": 1.2,

"stroke-dasharray": "4 4",

}),

);

const label = createSvgElement("text", {

x,

y: padding.top + 12 + index * 12,

"text-anchor": "middle",

class: "marker-label",

});

label.textContent = marker.label;

svg.appendChild(label);

});

}

series.forEach((entry) => {

const pathData = entry.values

.map((value, index) => {

const x = padding.left + innerWidth * normalizeLogFrequency(FREQUENCIES[index]);

const y =

padding.top +

innerHeight * (1 - (value - yRange.min) / (yRange.max - yRange.min));

return `${index === 0 ? "M" : "L"} ${x.toFixed(2)} ${clamp(y, padding.top, height - padding.bottom).toFixed(2)}`;

})

.join(" ");

svg.appendChild(

createSvgElement("path", {

d: pathData,

fill: "none",

stroke: entry.color,

"stroke-width": entry.label === "Combined" ? 3.4 : 2.4,

"stroke-linecap": "round",

"stroke-linejoin": "round",

opacity: options.showLegend ? 0.9 : 1,

}),

);

});

const xLabel = createSvgElement("text", {

x: padding.left + innerWidth / 2,

y: height - 4,

"text-anchor": "middle",

class: "axis-label",

});

xLabel.textContent = "Frequency (Hz)";

svg.appendChild(xLabel);

const yLabel = createSvgElement("text", {

x: 16,

y: padding.top + innerHeight / 2,

transform: `rotate(-90 16 ${padding.top + innerHeight / 2})`,

"text-anchor": "middle",

class: "axis-label",

});

yLabel.textContent = "Magnitude (dB)";

svg.appendChild(yLabel);

if (options.showLegend) {

const legendGroup = createSvgElement("g", {});

series.forEach((entry, index) => {

const x = padding.left + 10;

const y = padding.top + 14 + index * 14;

legendGroup.appendChild(

createSvgElement("line", {

x1: x,

y1: y,

x2: x + 18,

y2: y,

stroke: entry.color,

"stroke-width": 3,

"stroke-linecap": "round",

}),

);

const label = createSvgElement("text", {

x: x + 24,

y: y + 4,

class: "legend-label",

});

label.textContent = entry.label;

legendGroup.appendChild(label);

});

svg.appendChild(legendGroup);

}

}

function playNoisePreview() {

const context = ensureAudioContext();

stopAudio();

const buffer = context.createBuffer(1, context.sampleRate * 2, context.sampleRate);

const samples = buffer.getChannelData(0);

for (let index = 0; index < samples.length; index += 1) {

samples[index] = Math.random() * 2 - 1;

}

const source = context.createBufferSource();

source.buffer = buffer;

source.loop = true;

connectModelledAudio(source, readParameters());

source.start();

audioState.source = source;

setAudioStatus("Playing looped noise through the current outer-ear model.");

}

function playToneSweepPreview() {

const context = ensureAudioContext();

stopAudio();

const oscillator = new OscillatorNode(context, { type: "sine", frequency: 180 });

oscillator.frequency.exponentialRampToValueAtTime(16000, context.currentTime + 5);

connectModelledAudio(oscillator, readParameters());

oscillator.start();

oscillator.stop(context.currentTime + 5);

oscillator.addEventListener("ended", () => {

cleanupAudioGraph();

setAudioStatus("Sweep finished.");

});

audioState.source = oscillator;

setAudioStatus("Playing a logarithmic sweep through the current outer-ear model.");

}

async function handleAudioUpload(event) {

const file = event.target.files?.[0];

if (!file) {

audioState.uploadedBuffer = null;

dom.playFileButton.disabled = true;

setAudioStatus("Audio upload cleared.");

return;

}

try {

const context = ensureAudioContext();

const arrayBuffer = await file.arrayBuffer();

audioState.uploadedBuffer = await context.decodeAudioData(arrayBuffer.slice(0));

dom.playFileButton.disabled = false;

setAudioStatus(`Loaded "${file.name}". Ready to preview.`);

} catch (error) {

audioState.uploadedBuffer = null;

dom.playFileButton.disabled = true;

setAudioStatus("Could not decode that audio file.");

}

}

function playUploadedAudio() {

if (!audioState.uploadedBuffer) {

return;

}

const context = ensureAudioContext();

stopAudio();

const source = context.createBufferSource();

source.buffer = audioState.uploadedBuffer;

connectModelledAudio(source, readParameters());

source.start();

source.addEventListener("ended", () => {

cleanupAudioGraph();

setAudioStatus("Uploaded audio finished.");

});

audioState.source = source;

setAudioStatus("Playing the uploaded audio through the current outer-ear model.");

}

function connectModelledAudio(sourceNode, parameters) {

const context = ensureAudioContext();

const model = computeModel(parameters);

const reflector = model.metrics.reflector;

const concha = model.metrics.concha;

const canal = model.metrics.canal;

const reflection = model.metrics.reflection;

const shelf = new BiquadFilterNode(context, {

type: "highshelf",

frequency: clamp(reflector.breakHz, 80, context.sampleRate * 0.45),

gain: reflector.maxGainDb * 0.7,

});

const conchaPeak = new BiquadFilterNode(context, {

type: "peaking",

frequency: clamp(concha.helmholtzHz, 80, context.sampleRate * 0.45),

Q: concha.q,

gain: concha.gainDb,

});

const canalPeak = new BiquadFilterNode(context, {

type: "peaking",

frequency: clamp(canal.f1, 80, context.sampleRate * 0.45),

Q: canal.q1,

gain: canal.gain1Db,

});

const canalNotch = new BiquadFilterNode(context, {

type: "notch",

frequency: clamp(canal.f2, 80, context.sampleRate * 0.45),

Q: canal.q2,

});

const canalPeak3 = new BiquadFilterNode(context, {

type: "peaking",

frequency: clamp(canal.f3, 80, context.sampleRate * 0.45),

Q: canal.q3,

gain: canal.gain3Db,

});

const mainGain = new GainNode(context, { gain: 0.52 });

const dryGain = new GainNode(context, { gain: 1 });

const delayNode = new DelayNode(context, {

delayTime: clamp(

millimetersToMeters(reflection.effectivePathMm) / SPEED_OF_SOUND,

0,

0.08,

),

maxDelayTime: 0.1,

});

const wetGain = new GainNode(context, {

gain: reflection.effectiveStrength * 0.62,

});

const lowpass = new BiquadFilterNode(context, {

type: "lowpass",

frequency: 12000,

Q: 0.7,

});

const chainNodes = [sourceNode, shelf, conchaPeak, canalPeak, canalNotch, canalPeak3];

if (parameters.iec318Enabled) {

chainNodes.push(

new BiquadFilterNode(context, {

type: "peaking",

frequency: 220,

Q: 0.85,

gain: 1.4,

}),

new BiquadFilterNode(context, {

type: "peaking",

frequency: 2000,

Q: 1.05,

gain: 2.3,

}),

);

}

for (let index = 0; index < chainNodes.length - 1; index += 1) {

chainNodes[index].connect(chainNodes[index + 1]);

}

const outputNode = chainNodes[chainNodes.length - 1];

outputNode.connect(dryGain);

outputNode.connect(delayNode);

delayNode.connect(lowpass);

lowpass.connect(wetGain);

dryGain.connect(mainGain);

wetGain.connect(mainGain);

mainGain.connect(context.destination);

audioState.sourceNodes = [sourceNode];

audioState.activeNodes = [

shelf,

conchaPeak,

canalPeak,

canalNotch,

canalPeak3,

dryGain,

delayNode,

wetGain,

lowpass,

mainGain,

...chainNodes.slice(6),

];

}

function stopAudio() {

if (audioState.source) {

try {

audioState.source.stop();

} catch (error) {

// Source may already be stopped.

}

}

cleanupAudioGraph();

setAudioStatus("Audio idle.");

}

function cleanupAudioGraph() {

if (audioState.source) {

try {

audioState.source.disconnect();

} catch (error) {

// Ignore disconnect errors during cleanup.

}

}

audioState.activeNodes.forEach((node) => {

try {

node.disconnect();

} catch (error) {

// Ignore disconnect errors during cleanup.

}

});

audioState.source = null;

audioState.sourceNodes = [];

audioState.activeNodes = [];

}

function ensureAudioContext() {

if (!audioState.context) {

audioState.context = new (window.AudioContext || window.webkitAudioContext)();

}

if (audioState.context.state === "suspended") {

audioState.context.resume();

}

return audioState.context;

}

function setAudioStatus(message) {

dom.audioStatus.textContent = message;

}

function makePeakingFilter(frequency, q, gainDb) {

const omega = (2 * Math.PI * frequency) / ANALYSIS_SAMPLE_RATE;

const alpha = Math.sin(omega) / (2 * q);

const amplitude = Math.pow(10, gainDb / 40);

const cosine = Math.cos(omega);

const b0 = 1 + alpha * amplitude;

const b1 = -2 * cosine;

const b2 = 1 - alpha * amplitude;

const a0 = 1 + alpha / amplitude;

const a1 = -2 * cosine;

const a2 = 1 - alpha / amplitude;

return normalizeBiquad({ b0, b1, b2, a0, a1, a2 });

}

function makeNotchFilter(frequency, q, depthDb) {

const omega = (2 * Math.PI * frequency) / ANALYSIS_SAMPLE_RATE;

const alpha = Math.sin(omega) / (2 * q);

const cosine = Math.cos(omega);

const notch = normalizeBiquad({

b0: 1,

b1: -2 * cosine,

b2: 1,

a0: 1 + alpha,

a1: -2 * cosine,

a2: 1 - alpha,

});

notch.mix = clamp(depthDb / 12, 0.2, 0.85);

return notch;

}

function normalizeBiquad(coefficients) {

return {

b0: coefficients.b0 / coefficients.a0,

b1: coefficients.b1 / coefficients.a0,

b2: coefficients.b2 / coefficients.a0,

a1: coefficients.a1 / coefficients.a0,

a2: coefficients.a2 / coefficients.a0,

mix: coefficients.mix ?? 1,

};

}

function biquadMagnitudeAtFrequency(filter, frequency) {

const omega = (2 * Math.PI * frequency) / ANALYSIS_SAMPLE_RATE;

const cos1 = Math.cos(omega);

const sin1 = Math.sin(omega);

const cos2 = Math.cos(2 * omega);

const sin2 = Math.sin(2 * omega);

const numeratorReal = filter.b0 + filter.b1 * cos1 + filter.b2 * cos2;

const numeratorImaginary = -filter.b1 * sin1 - filter.b2 * sin2;

const denominatorReal = 1 + filter.a1 * cos1 + filter.a2 * cos2;

const denominatorImaginary = -filter.a1 * sin1 - filter.a2 * sin2;

const numeratorMagnitude = Math.sqrt(

numeratorReal ** 2 + numeratorImaginary ** 2,

);

const denominatorMagnitude = Math.sqrt(

denominatorReal ** 2 + denominatorImaginary ** 2,

);

const rawMagnitude = numeratorMagnitude / denominatorMagnitude;

if (filter.mix === 1) {

return rawMagnitude;

}

return (1 - filter.mix) + filter.mix * rawMagnitude;

}

function findExtremum(frequencies, values, mode) {

let bestIndex = 0;

let bestValue = values[0];

values.forEach((value, index) => {

if ((mode === "max" && value > bestValue) || (mode === "min" && value < bestValue)) {

bestIndex = index;

bestValue = value;

}

});

return { frequency: frequencies[bestIndex], value: bestValue };

}

function formatParameterValue(key, value) {

switch (key) {

case "conchaVolumeCc":

case "middleCavityVolumeCc":

return `${value.toFixed(1)} cc`;

case "reflectionStrength":

return value.toFixed(2);

case "azimuthDeg":

case "elevationDeg":

return `${value.toFixed(0)} deg`;

case "middleInertance":

case "middleCompliance":

case "jointCompliance":

return `${value.toFixed(6)} m.u.`;

case "middleResistance":

case "cochlearLoad":

return `${value.toFixed(2)} m.u.`;

default:

return `${value.toFixed(1)} mm`;

}

}

function formatFrequency(frequency) {

if (frequency >= 1000) {

const precision = frequency < 10000 ? 2 : 1;

return `${(frequency / 1000).toFixed(precision)} kHz`;

}

return `${frequency.toFixed(0)} Hz`;

}

function formatFrequencyTick(frequency) {

if (frequency >= 1000) {

return `${frequency / 1000}k`;

}

return `${frequency}`;

}

function formatDb(value) {

return `${value >= 0 ? "+" : ""}${value.toFixed(1)} dB`;

}

function formatSignedHz(value) {

return `${value >= 0 ? "+" : ""}${value.toFixed(0)} Hz`;

}

function logSpace(minimum, maximum, count) {

const output = [];

const start = Math.log10(minimum);

const end = Math.log10(maximum);

for (let index = 0; index < count; index += 1) {

const ratio = index / (count - 1);

output.push(10 ** interpolate(start, end, ratio));

}

return output;

}

function normalizeLogFrequency(frequency) {

return (

(Math.log10(frequency) - Math.log10(FREQUENCIES[0])) /

(Math.log10(FREQUENCIES[FREQUENCIES.length - 1]) - Math.log10(FREQUENCIES[0]))

);

}

function createSvgElement(tagName, attributes) {

const element = document.createElementNS("http://www.w3.org/2000/svg", tagName);

Object.entries(attributes).forEach(([key, value]) => {

element.setAttribute(key, value);

});

return element;

}

function getInput(key) {

return document.querySelector(`[data-param="${key}"]`);

}

function dbToMagnitude(value) {

return 10 ** (value / 20);

}

function magnitudeToDb(value) {

return 20 * Math.log10(Math.max(value, 1e-6));

}

function millimetersToMeters(value) {

return value / 1000;

}

function cubicCentimetersToCubicMeters(value) {

return value * 1e-6;

}

function degreesToRadians(value) {

return (value * Math.PI) / 180;

}

function interpolate(start, end, ratio) {

return start + (end - start) * ratio;

}

function clamp(value, minimum, maximum) {

return Math.min(maximum, Math.max(minimum, value));

}